[image: image1.png]



Weekly Safety Meeting

Company Name

Project Name

Supervisor


Date

	
	
	
	


NFPA 70E
1. Introduction to NFPA 70E

· What is NFPA 70E?
The NFPA 70E standard outlines safe practices to protect employees from electrical hazards such as arc flash, electric shock, and arc blast.

· Why is it Important?
Electrical hazards can cause severe injuries or fatalities. NFPA 70E helps reduce these risks through hazard assessments, risk analysis, and proper safety practices.

2. Recognizing Electrical Hazards

· Types of Hazards:

· Electric Shock: Can occur when electrical current passes through the body.

· Arc Flash: A rapid release of electrical energy creating intense heat, light, and pressure.

· Arc Blast: The explosive energy from an arc flash that can send molten metal, debris, and create extreme pressure.

· Identifying Hazard Levels:
NFPA 70E requires assessing each work area for potential hazards and using a Risk Assessment to determine the level of risk associated with each task.

3. Establishing Electrically Safe Work Conditions

· Lockout/Tagout (LOTO):
De-energize equipment and follow LOTO procedures to prevent accidental energizing.

· Testing Equipment:
Use appropriate testers to verify the absence of voltage before beginning any work. Always test your tester on a known live circuit before and after verifying.

· Working at Safe Distances:
Maintain proper distances from live equipment based on the voltage rating. NFPA 70E provides guidance on safe approach boundaries.
4. Personal Protective Equipment (PPE)

· PPE Categories:
PPE is divided into categories based on hazard levels, from basic protection to heavy-duty protection for high-risk environments.

· Common PPE for Electrical Work:

· Arc-rated clothing (shirts, pants, coveralls)

· Voltage-rated gloves and leather protectors

· Face shields and goggles for eye protection

· Hearing protection

· Arc flash suits for high-risk tasks

· Using PPE Properly:
Inspect PPE before each use and replace any damaged or expired equipment.
5. Training and Procedures

· Employee Training:
NFPA 70E requires that all employees exposed to electrical hazards receive training on safe work practices, proper PPE use, and emergency response.

· Documented Procedures:
Ensure all employees follow established procedures and that these are documented, updated, and accessible.
6. Emergency Response and Incident Reporting

· Emergency Procedures:
In case of an electrical incident, know the emergency response procedures, including first aid for shock or burns.

· Incident Reporting:
Report any near misses or incidents immediately to help prevent future accidents and to improve safety protocols.
7. Questions and Review

· Encourage employees to ask questions if they are unclear about any safety practices.

· Remind everyone of the importance of vigilance and adherence to NFPA 70E guidelines to prevent accidents.

Conclusion:
Electrical safety is a shared responsibility. NFPA 70E provides the foundation to ensure that everyone has the knowledge and equipment needed to stay safe around electrical systems.

Additional Topics and Meeting Attendees on back:
ADDITIONAL TOPICS COVERED: (I.E. Hazcom, Emergency Plan)
	1.



	2.



	3.




MEETING ATTENDEES:

PRINT NAME



SIGN NAME


EMPLOYEE NUMBER
	1.


	
	

	2.


	
	

	3.


	
	

	4.


	
	

	5.


	
	

	6.


	
	

	7.


	
	

	8.


	
	

	9.


	
	

	10.


	
	

	11.


	
	

	12.


	
	


NFPA 70E
This Toolbox Meeting is intended as an overview of key points regarding this subject and is not intended to be complete training in accordance with any regulatory standards.

[image: image1.png]