| MOVING EQUIPMENT TOOLBOX TALK |  |
| --- | --- |

Never assume an equipment operator sees you. Stay out of blind spots, swing areas, travel paths, and suspended-load zones.

| Date | Project / Location | Instructor |
| --- | --- | --- |

Introduction

Open with: “Today’s safety topic is moving equipment. Cranes, dozers, excavators, trucks, and other equipment can create serious struck-by, caught-between, and electrical hazards.”

Key Points: Assume the operator does not see you. Stay clear of blind spots, backing equipment, swing areas, and suspended loads.

Why It Matters

Construction equipment is heavy, powerful, and often operates in noisy, congested areas where workers may be difficult for operators to see or hear.

Warning horns can be lost in general jobsite noise.

Backing equipment, swinging counterweights, shifting loads, and sudden turns can place workers in danger quickly.

Quick Engagement Question: “Where are the biggest blind spots or moving-equipment hazards on our jobsite today?”

Recognize the Hazards

| Worker Exposure Blind spots around equipment. Backing and turning equipment. Swinging counterweights and pinch points. Walking beside moving equipment. | Other Serious Hazards Suspended loads from cranes or hoists. Riding on running boards or loaded trucks. Overhead power-line contact. Servicing or cleaning equipment while it is operating. |
| --- | --- |

Mini Scenario: “An excavator is backing toward your work area and you cannot make eye contact with the operator. What should you do before entering or crossing the equipment’s path?”

Safe Work Practices

| Stay Visible & Clear Never assume the operator sees you. Keep clear when equipment is backing. Use established walkways and avoid shortcuts. | Avoid Pinch & Struck-By Hazards Stay out of counterweight swing areas. Never walk alongside moving equipment. Stay out from under suspended loads. | Riding & Electrical Safety Never ride on running boards or on top of loaded trucks. Keep all body parts inside transport vehicles. If equipment contacts a power line, stay away from the frame and load cables. |
| --- | --- | --- |

Equipment Servicing

Remember: Never lubricate, clean, adjust, or work on a machine while it is operating. Stop the machine before work begins, and replace any guard that was removed as soon as the work is complete.

Close

Closing Message: “Treat every piece of moving equipment as if the operator cannot see you. Stay visible, stay out of the line of fire, and never put yourself between moving equipment and a fixed object.”

Toolbox Talk Attendance

Topic: Moving Equipment

| Date: | Project / Location: | Instructor: |
| --- | --- | --- |

By signing below, participants acknowledge attendance at this toolbox talk.

| # | Printed Name | Signature | Date |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |