| CONCRETE & MASONRY TOOLBOX TALK |  |
| --- | --- |

Concrete and masonry work can expose employees to heavy loads, rebar, suspended buckets, formwork, wall collapse, and electrical hazards.

| Date | Project / Location | Instructor |
| --- | --- | --- |

Introduction

Open with: “Today’s safety topic is concrete and masonry construction—because rebar, suspended buckets, formwork, and unsupported walls can cause serious injuries.”

Key Points: Verify structural capacity. Guard rebar. Stay clear of concrete buckets. Support formwork and brace walls.

Why It Matters

Do not load concrete structures until the employer confirms they can support the load using information from someone qualified in structural design.

Protruding rebar can impale workers, and elevated concrete buckets create struck-by and crushing hazards.

Formwork and masonry walls can fail if they are not properly supported and braced.

Quick Engagement Question: “What is one concrete or masonry hazard on this job that could seriously injure someone if it is not controlled?”

Recognizing Hazards & Risks

Look for hazards before work begins and whenever site conditions change.

| Concrete / Rebar Hazards Unconfirmed structural load capacity Unguarded protruding rebar Workers under raised/lowered buckets Riding on or in concrete buckets | Formwork / Masonry Hazards Inadequate formwork support Unbraced masonry walls over 8 ft. Missing limited access zones Wet concrete near electrical wires |
| --- | --- |

Mini Scenario: “A masonry wall over 8 feet high is not permanently supported, and workers are entering the limited access zone. What must be corrected before work continues?”

Safe Work Practices

Use the required controls every time. Do not take shortcuts around suspended loads, rebar, formwork, or unsupported walls.

| Before Work Confirm load capacity Guard protruding rebar Verify formwork support | During Work Stay clear of concrete buckets Reduce exposure below bucket routes Maintain wall bracing/access controls | Electrical Awareness Wet concrete conducts electricity Use caution with bull floats Keep handles away from wires |
| --- | --- | --- |

Resources & Close

Questions? Stop and ask your supervisor or a qualified structural-design person before proceeding.

Supervisor / qualified structural-design person

Closing Message: “Control concrete and masonry hazards before work starts.”

Toolbox Talk Attendance

Topic: Concrete & Masonry Construction

| Date: | Project / Location: | Instructor: |
| --- | --- | --- |

By signing below, participants acknowledge attendance at this toolbox talk.

| # | Printed Name | Signature | Date |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |