| ILLUMINATION TOOLBOX TALK |  |
| --- | --- |

Good illumination helps workers see walking surfaces, tools, materials, and hazards clearly. Poor lighting can contribute to trips, falls, tool injuries, and unsafe temporary electrical conditions.

| Date | Project / Location | Instructor |
| --- | --- | --- |

Introduction

Open with: “Today’s safety topic is illumination—making sure we can clearly see the work, the path, and the hazards around us.” Reinforce that adequate lighting is a basic safety control, not just a convenience.

Key Points: Construction areas need adequate natural or artificial lighting. Temporary lighting and wiring must be installed and maintained safely.

Why It Matters

Poor lighting can hide changes in elevation, debris, cords, edges, equipment, and other hazards. It can also make layout work and tool use more difficult and less accurate.

Construction areas, stairs, ramps, corridors, storage areas, shops, and offices need adequate illumination.

General construction areas require at least 5 foot-candles of illumination.

Quick Engagement Question: “Where on this job could poor lighting create the greatest risk for a trip, fall, or tool-related injury?”

Recognizing Lighting Hazards

Before starting work, look for shadows, glare, damaged lights, poor placement, or unsafe temporary wiring that could create a hazard.

| Hazards to Watch For Dark stairs, ramps, corridors, or work areas Damaged or unguarded lamps Cords exposed to traffic or damage | Safe Lighting Practices Add or reposition lighting as needed Protect lamps from contact and breakage Use hard or extra-hard usage cords Protect temporary wiring from damage |
| --- | --- |

Mini Scenario: “You are preparing to use a cutting tool where shadows make the layout marks hard to see. What should you do before starting?”

Temporary Lighting & Wiring

Temporary lighting must be set up so it does not create an electrical, fire, contact, or trip hazard.

| Lighting Guard lamps from breakage Ground metal-case sockets Provide enough light for the task | Cords & Wiring Protect wiring from traffic and damage Avoid overloaded or undersized circuits Do not suspend lights by cords unless designed for it | Closeout Remove temporary wiring when construction is complete Report damaged lighting or wiring immediately |
| --- | --- | --- |

Resources & Close

If lighting is inadequate or temporary wiring appears unsafe, stop and notify your supervisor before continuing the work.

Closing Message: “If lighting is inadequate, stop and improve it before continuing.”

Toolbox Talk Attendance

Topic: Illumination

| Date: | Project / Location: | Instructor: |
| --- | --- | --- |

By signing below, participants acknowledge attendance at this toolbox talk.

| # | Printed Name | Signature | Date |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |