| SCAFFOLD SAFETY TOOLBOX TALK |  |
| --- | --- |

Scaffolds expose workers to falls, falling objects, and collapse. Inspect before use and follow safe loading, access, and rolling-scaffold practices.

| Date | Project / Location | Instructor |
| --- | --- | --- |

Introduction

Open with: “Before you trust a scaffold with your life, make sure it is safe.” Do not use damaged, unstable, overloaded, or unsafe scaffolds.

Key Points: Inspect before use. Keep platforms clear. Do not overload. Use safe access. Follow rolling scaffold precautions.

Why It Matters

Scaffold injuries can result from failed planking or supports, slips, falls, and falling objects. Conditions can change from weather, loading, or equipment contact.

Inspect before each use and watch for changing conditions.

Never assume a scaffold is safe because it was safe yesterday.

Quick Engagement Question: “What is one scaffold condition that would make you stop and not use it until it is corrected?”

What to Check

Check guardrails, connectors, fastenings, footings, tie-ins, bracing, platforms, access, and stability. Keep platforms secure and free of debris and slip hazards.

| Hazards to Watch For Loose or damaged components Overloaded platforms Debris, ice, or trip hazards | Safe Scaffold Practices Inspect before use Keep platforms secure and clear Use proper access and bracing |
| --- | --- |

Mini Scenario: “A rolling scaffold has loose material on the platform and one caster brake is not engaged. What must be corrected before use or movement?”

Rolling Scaffold Safety

Do not ride rolling scaffolds. Remove loose material before moving, use enough help, avoid holes and overhead obstructions, and lock caster brakes when stationary. Stabilize the scaffold when required.

| Before Use Inspect all components Check platform and access Verify stable footing and bracing | While Working Do not overload Keep the platform clear Control hoisted material with a tagline | Rolling Scaffolds Never ride while moving Lock casters when stationary Clear the route before moving |
| --- | --- | --- |

Resources & Close

Do not overload scaffolds. Control hoisted material with a tagline, use safe access, and follow jobsite scaffold procedures.

Closing Message: “Inspect before use, keep the scaffold stable and clear, and never take chances with a condition that could cause a fall or collapse.”

Toolbox Talk Attendance

Topic: Scaffold Safety

| Date: | Project / Location: | Instructor: |
| --- | --- | --- |

By signing below, participants acknowledge attendance at this toolbox talk.

| # | Printed Name | Signature | Date |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |