| FORKLIFT SAFETY TOOLBOX TALK |  |
| --- | --- |

Forklifts are powerful, compact machines. Safe operation depends on inspection, controlled travel, proper load handling, and constant awareness of pedestrians.

| Date | Project / Location | Instructor |
| --- | --- | --- |

Introduction

Common forklift incidents include striking people or objects, collisions, loading dock falls, operator injuries, and tip-overs from excessive speed or improper loads.

Key Points: Inspect before use. Stay within rated capacity. Keep people clear. Control speed and load height.

Pre-Operational Inspection

Before operating, inspect brakes, steering, horn/alarms, warning lights, tires, mast, forks, hoses, hydraulics, leaks, load limits, and safety devices.

Document defects and notify maintenance or supervision.

Never drive a forklift that is in need of repair.

Quick Engagement Question: “What is the first thing you should do before operating a forklift at the start of your shift?”

Mounting, Dismounting & Travel

Face the vehicle and use three points of contact. Never jump off. Travel at a controlled speed, look in the direction of travel, keep hands and legs inside, and keep forks about 2–4 inches above the floor.

| Common Forklift Hazards Struck-by incidents Tip-overs Loading dock falls Blocked visibility | Pre-Use Checks Brakes, steering, horn Tires, mast, forks Leaks and hydraulics Load limits / safety devices |
| --- | --- |

Mini Scenario: “During your pre-use inspection you find a hydraulic leak. What should happen before the forklift is operated?”

Pedestrians & Load Control

Pedestrians have the right-of-way. Slow down and sound the horn at blind corners. Do not carry riders or lift/lower loads while traveling. Drive in reverse if the load blocks forward vision.

| Before Moving Complete inspection Confirm clear route Set forks low | While Traveling Control speed Watch pedestrians Sound horn at blind areas | Never Do This Use defective equipment Carry riders Exceed rated capacity |
| --- | --- | --- |

Safe Travel Reminders

Start and stop smoothly. Use caution on curves, inclines, slippery floors, bumps, holes, and overhead obstacles. Keep loads within rated capacity and maintain a safe following distance.

Keep the mast slightly tilted back while traveling.

Watch for overhead obstacles and floor hazards.

Maintain at least three forklift lengths of following distance.

Closing Message: “Inspect before use, control your speed and load, watch for pedestrians, and never operate a forklift that needs repair.”

Toolbox Talk Attendance

Topic: Forklift Safety

| Date: | Project / Location: | Instructor: |
| --- | --- | --- |

By signing below, participants acknowledge attendance at this toolbox talk.

| # | Printed Name | Signature | Date |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |