| LOCKOUT/TAGOUT (LOTO) TOOLBOX TALK |  |
| --- | --- |

Lockout/Tagout (LOTO) protects workers from unexpected start-up or the release of hazardous energy during servicing and repair. De-energize, lock/tag, control stored energy, and verify before work begins.

| Date | Project / Location | Instructor |
| --- | --- | --- |

Introduction

Open with: “Today’s safety topic is Lockout/Tagout. Turning equipment off is not enough—we must isolate every hazardous energy source before servicing or repair.”

Key Points: Isolate energy. Apply your personal lock and tag. Verify zero energy.

Why It Matters

Unexpected energization can cause electrocution, crushing, burns, amputations, or fatal injuries.

Never rely only on a stop button, switch, or control circuit.

Each authorized worker controls their own lock and key.

Quick Engagement Question: “How do you know equipment is truly safe to work on—not just turned off?”

Recognizing Hazardous Energy & LOTO Triggers

Identify all energy sources and stored energy before work starts.

| Common Energy Sources Electrical Hydraulic / pneumatic Mechanical / gravity Thermal / stored energy | LOTO Triggers Servicing or repair Removing guards Clearing jams Entering a danger zone |
| --- | --- |

Mini Scenario (2–3 min): “A conveyor is switched off, but the disconnect is not locked. A worker reaches in to clear a jam. What could happen, and what must be done first?”

LOTO Procedure & Worker Responsibilities

Follow the written equipment-specific procedure. If an isolation point or energy source is unclear, stop and ask.

| Before Lockout Identify energy sources Notify affected employees Shut down and isolate | Apply & Verify Apply personal lock/tag Control stored energy Try-start / verify zero energy | Restore Safely Replace guards Clear employees Each worker removes own lock |
| --- | --- | --- |

Key Requirements & Close

Authorized employees must be trained and follow the company LOTO policy and equipment-specific procedures.

Never service equipment that has not been properly de-energized and locked/tagged out.

Do not remove another employee’s lock except through the company’s formal procedure.

When in doubt, stop work and verify the energy-control procedure.

Closing Message: “LOTO means isolate, lock/tag, control stored energy, and verify. If zero energy has not been confirmed, the work does not begin.”

Toolbox Talk Attendance

Topic: Lockout/Tagout (LOTO)

| Date: | Project / Location: | Instructor: |
| --- | --- | --- |

By signing below, participants acknowledge attendance at this toolbox talk.

| # | Printed Name | Signature | Date |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |